LA-ICP-MS full method and representative TRAs

Laser ablation analysis of sulfides was undertaken at Cardiff University using an ESI P213 Laser Ablation System coupled to a Thermo X Series 2 ICP-MS. Spot analyses (55 µm) were carried out on sulfides from the Muratdere deposit (n=22). The laser was operated at 15 Hz pulse rate with a dwell time of 40 seconds, and a gas blank measured for 30 seconds prior to acquisition. Line analyses were carried out on sulfides from the Skouries and El Teniente deposits (n=343). A 55 μm beam diameter was used at a frequency of 10 Hz and ablation lasted between 80 and 120 s with a gas blank measured for 20 s prior to analysis. The beam followed a pre-selected pattern designed to sample the diameter of the sulfide crystal, with the sample translating at 6 μm/s relative to the laser.

SEM-EDS surveys were carried out prior to LA-ICP-MS analysis to identify surface micro-inclusions and only sulfides without visible micro-inclusions were analyzed. To mitigate the effect of ablating small inclusions beneath the surface, only flat regions of TRA spectra were selected for integration. Trace element abundance was measured using the following isotopes: 57Fe, 59Co, 61Ni, 65Cu, 66Zn, 75As, 77Se, 99Ru, 103Rh, 105Pd, 108Pd, 109Ag, 111Cd, 121Sb, 125Te, 185Re, 189Os, 193Ir, 195Pt, 197Au, 206Pb and 209Bi. Dwell times of 2 ms were used for major elements, 10 ms for semi-metals and 20 ms for PGEs. Measured isotopes were selected to avoid isobaric and polyatomic interferences, and 99Ru, 101Ru, 103Rh, 105Pd, 106Pd and 108Pd interferences were corrected. Limits of detection for all elements analyzed are given in Table 2 and appendix A1.

Instrument calibration was performed using a series of five synthetic Ni-Fe-S quenched sulfide standards (composition of standards is given in Prichard et al., 2013). The standards produce five-point calibration curves for S, Ni, and Fe and three-point calibration curves for PGE, Ag, Cd, Re, Au, Cu, Co, Zn, and semi-metals. Gas blank subtraction and internal standard corrections were carried out using Thermo Plasmalab software and 33S was used as an internal standard for trace element calibration. Accuracy and precision for Au and PGE were checked by analyzing the CANMET Po727 Memorial standard as an unknown against the Cardiff quenched sulfide standards at the beginning and end of each sample run. Accuracy was very good to excellent for all elements (Piercey, 2014) and precision was good for all elements with an average 2σ of 4.4 ppm. Full standard analyses are given in appendix A1.

Below are time resolved analyses (TRA) of key sulfides from the three deposits in this study, illustrating the distirbution of elements within the sulfides.

El Teniente chalcopyrite

El Teniente chalcopyrite