# Mass balance calculations

Mass balance calculations were performed for the samples analyzed in this study to see if the majority of the elements of interest are present in the sulfides, or if they are present in non-sulfide accessory minerals. The metal concentration of a whole rock sample was estimated using the concentration of the elements of interest in sulfides and the modal percentage of sulfides in the corresponding whole rock sample, using the equation below (here using Au):

XAuWR = ρsul/ρwr*Y*XAuSul

Where XAuWR is the concentration of Au, or the element of interest, in the whole rock; ρsul is the density of the sulfide; ρwr is the density of the whole rock; Y is the estimated volume percentage of sulfide in the sample as a fraction and XAuSul is the concentration of Au in the sulfide as a weight fraction.

These calculated values were then compared to the measured whole rock concentrations and the percentage relative difference calculated according to the equation below:

%RD = [(XAuMeas-XAuCalc)/XAuMeas]*100

Where XAuMeas is the measured whole rock Au concentration for that sample and XAuCalc is the calculated Au concentration for that sample.

The calculation was carried out with Cu first to check the accuracy of the method as all Cu in PCDs is hosted in sulfides. Cu concentration was predicted fairly well by the equation in samples where Cu sulfides were analyzed, with percentage relative differences of between -17% and 9%, with the biggest source of error being the estimation of percentage volume of sulfides.

# Results

Gold has a relative difference between Au concentration calculated from sulfide measurement and measured Au concentration of 26 – 100%, showing that the Au concentration in sulfides is not a reliable indicator of Au concentration in whole rock. This means it is likely that the majority of Au in these deposits is hosted in accessory minerals such as electrum (AuxAg1-x) or as native gold. Samples from the Skouries deposit have a median RD of 100%, meaning that most of the Au in these samples is likely in electrum. This is supported by SEM-EDS surveys (McFall et al 2018) which show abundant electrum and native gold as accessory minerals in mineralized vein sets. Muratdere also has high RD with a median RD of 93%, while El Teniente has a median RD of 76% which suggests that sulfide has more control on Au concentration in El Teniente than Muratdere or Skouries. It is worth noting, however, that the overall Au concentrations in El Teniente are significantly lower than those in Muratdere and Skouries.

Although it is present in up to 100s ppm in sulfides, silver also has high RD. Silver in El Teniente has a median RD of 19%, while Ag in Muratdere has a median RD of 70% and in Skouries the median RD is 87%. This suggests that the majority of the Ag in El Teniente is hosted in sulfides, while that in Muratdere and Skouries is hosted in accessory minerals. Accessory hessite (Ag2Te) and electrum (AuxAg1-x) has been documented in Skouries (McFall et al., 2018), and is likely present in Muratdere as well.

Te and Bi have large RDs in Skouries, with median RDs of 89% and 93% respectively whereas in El Teniente and Muratdere the RD of Bi and Te are both below 30%. This suggests that most of the Bi and Te in El Teniente and Muratdere are in sulfides, while those in Skouries are in accessory minerals. This is supported by mineralogical surveys of Skouries which showed Bi-Te accessory minerals are present in almost all mineralized samples (McFall et al., 2018). Most of the calculated whole rock Se concentrations are greater than the measured concentrations in all three deposits which means that all the Se is hosted in the sulfides.

Graphs showing the calculated concentration of the elements of interest against the measured whole rock value – when the measured value is higher it is likely these elements are in accessory minerals.

Table of relative difference results for mass balance calculations.

| Sample | Lithology | Sulphides present | Cu RD | Au RD | Ag RD | Te RD | Se RD | Bi RD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KM ET1 | Andesite | cp py | 4 | 89 | 50 | 38 | -3 | 40 |
| KM ET2 | Porphyry | cp mo py | -9 | 26 | 13 | -339 | -319 | -1228 |
| KM ET3 | Porphyry | cp mo | 39 | 70 | 90 | -91 | -76 | 100 |
| KM ET4 | Andesite | bn cp | -17 | 83 | -82 | 27 | 42 | -10 |
| KM ET5 | Porphyry | cp py | 8 | 43 | 24 | 36 | -40 | 26 |
| KM ET6 | Porphyry | cp | -3 | 92 | -47 | 53 | -66 | -12 |
| MD1 | Granodiorite | mo py | 99 | 96 | 95 | 6 | -149 | -8 |
| MD10 | Granodiorite | ga py sp | 97 | 99 | 87 | 96 | 15 | 14 |
| MD2 | Granodiorite | cp mo py | -7 | 39 | 4 | -309 | -533 | -407 |
| MD3 | Granodiorite | mo | 93 | 54 | 46 | 45 | -333 | -19 |
| MD6 | Granodiorite | cp mo py | 9 | 90 | 70 | -117 | -210 | -22 |
| KM SK1 | Syenite | py | 100 | 94 | 95 | 32 | -55 | 92 |
| KM SK10 | Syenite | bn cp | 7 | 98 | 61 | 68 | -29 | 17 |
| KM SK11 | Syenite | cp py | -6 | 100 | 84 | 89 | -52 | 99 |
| KM SK12 | Syenite | cp | 30 | 100 | 85 | 90 | 3 | 87 |
| KM SK13 | Syenite | cp | -9 | 100 | 86 | 91 | -117 | 97 |
| KM SK2 | Syenite | cp | -1 | 100 | 82 | 96 | -33 | 96 |
| KM SK3 | Syenite | cp | 7 | 100 | 70 | 0 | -42 | 50 |
| KM SK4 | Syenite | cp py | -5599 | 95 | 97 | 96 | 62 | 93 |
| KM SK5 | Syenite | cp | 41 | 99 | 88 | 59 | 7 | 52 |
| KM SK6 | Syenite | cp py | 5 | 99 | 87 | 54 | -36 | 37 |
| KM SK7 | Syenite | cp py | -9 | 100 | 95 | 89 | 38 | 93 |
| KM SK8 | Syenite | cp | 2 | 100 | 96 | 91 | -28 | 99 |